# TU-100 Emulator Quick Reference Card

Valid Valve Configurations - Use these exact patterns to run the emulator

| Configuration | HP1 | HP2 | HP3 | HP4 | LRI | LRK | LRB | TXV | CTV | ACI | ACO | ACB |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cooling with TXV, Liquid Receiver and Accumulator | ON | OFF | OFF | ON | ON | ON | OFF | ON | OFF | ON | ON | OFF |
| Heating with TXV, Liquid Receiver and Accumulator | OFF | ON | ON | OFF | ON | ON | OFF | ON | OFF | ON | ON | OFF |
| Cooling with CTV and Accumulator (no LR) | ON | OFF | OFF | ON | OFF | OFF | ON | OFF | ON | ON | ON | OFF |
| Heating with CTV and Accumulator (no LR) | OFF | ON | ON | OFF | OFF | OFF | ON | OFF | ON | ON | ON | OFF |
| Cooling with TXV and Liquid Receiver (no AC) | ON | OFF | OFF | ON | ON | ON | OFF | ON | OFF | OFF | OFF | ON |
| Heating with TXV and Liquid Receiver (no AC) | OFF | ON | ON | OFF | ON | ON | OFF | ON | OFF | OFF | OFF | ON |
| Cooling with CTV only | ON | OFF | OFF | ON | OFF | OFF | ON | OFF | ON | OFF | OFF | ON |
| Heating with CTV only | OFF | ON | ON | OFF | OFF | OFF | ON | OFF | ON | OFF | OFF | ON |

Legend:

• Green (ON) = Valve is open / active

• Red (OFF) = Valve is closed / inactive

Valve Abbreviations:

HP1-HP4: High Pressure Valves | LRI: Liquid Receiver Inlet | LRK: Liquid Receiver King Valve

LRB: Liquid Receiver Bypass | TXV: Thermostatic Expansion Valve | CTV: Capillary Tube Valve

ACI: Accumulator Inlet | ACO: Accumulator Outlet | ACB: Accumulator Bypass